	The MACK Group, LLC Completes Asbestos Removal at Old Textile Mill

	The MACK Group, LLC completed a large asbestos abatement removal project at a former textile plant in Northern New Jersey.  The client wanted to fast track the asbestos removal in order to expedite the demolition of the facility. The MACK Group, LLC (MACK) was selected to perform the project based on their expert level of asbestos contractor services, their professional reputation, their attention to safety and schedule as well as their economical approach to asbestos removal services.

The asbestos removal project consisted of the asbestos abatement of 41,000 Square feet of roofing materials, 1,000 square feet of plaster, 200 square feet VAT and 138 linear feet of pipe insulation.  There was also some flooring tile to be removed.  These materials were located throughout the multi-story building complex. 
Prior to asbestos abatement activities, a 10 day notification was submitted which is a written notification of intention to demolish in accordance with 40 CFR 61.145 to 61.155 and N.J.A.C. 7:26-2.12(d) and (e).

Because the former plant had been vacant for a long period of time all utilities had been previously disconnected.  This necessitated that MACK utilize large onsite generators to supply all power for the HEPA negative air filtration units used inside work area containments.
Proper asbestos wetting, removal and handling techniques were employed to safely remove the materials. MACK used various techniques to remove the asbestos materials including wetting of all friable ACM and bagging the ACM using poly-wrapping.  Also utilized was the wetting and direct placement in lined (at least 6 mil thick liner) and sealed bulk containers. 
To remove the asbestos floor tiles and mastic, MACK used a combination of proprietary flooring removal machines that they own in their fleet of abatement equipment.   The flooring removal machines have zero emissions and fumes allowing for use in restricted ventilation areas.  These machines allowed MACK to easily remove the floor tile and mastic. 
In addition, MACK utilized manlifts, lulls and skid steers to facilitate the removal of the asbestos materials.  Over 15 licensed and skilled asbestos workers and supervisors were also engaged to complete the project.

The transportation of asbestos and ACM was in accordance with N.J.A.C. 7:26-3.5(d) and by registered New Jersey solid waste vehicles.  ACM was then properly disposed at a licensed landfill. 
The project was brought in ahead of schedule and without injury due to the level of skilled manpower and equipment, as well as corporate management oversight, that the MACK committed to the project.
Following asbestos abatement activities, the buildings were then demolished. 
About Us

	The MACK Group, LLC provides nationwide asbestos abatement, asbestos removal and demolition contractor services. Contractor services include asbestos abatement, asbestos removal, transite removal, asbestos roof removal, demolition, Terminator flooring removals, floor removal contractor services, Shot Blast/Blastrac services and fluorescent bulb and ballast recycling services.

The MACK Group

1500 Kings Hwy. N., Ste 209

Cherry Hill, NJ 08034

Tel:   973-759-5000

Fax:  973-759-5554

Web:   www.MACKgrp.com
For more information contact: 
Stephen King (973) 759-5000 ext. 302
SKing@mackgrp.com 


