


	[image: image1.jpg]® ¢

35-W1005P-02L-XUL
USTED  PRESSURE GAUGE FOR
346R FIRE pRoTECTON  P'SI

W\
\\ SERVICE
N




        [image: image2.jpg]35-W1005P-02L-XUL
PRESSURE GAUGE FOR

FIRE PROTECTION
SERVICE






	NEW!
Series 35W
Fire Protection, Sprinkler

 Service Gage



Dwyer Instruments, Inc., the preferred source for your complete range of practical and affordable instrumentation, is announcing the release of its NEW 35W Series Fire Protection, Sprinkler Service Gage.


The 35W Series Fire Protection Sprinkler Service Gage possesses psi scale with ±3-2-3% ASME B 40.1 Grade B accuracy. The 3.5” gages are Underwriters Laboratory listed and Factory Mutual approved for fire protection sprinkler service. This gage features a corrosion-resistant ABS case and heat-resistant polycarbonate push-in window. In addition, it also features a movement that is designed to provide shock and vibration resistance that results in a superior performance and extended gage life. The 0-300 psi pressure range is used on “wet” systems where water is available to the sprinkler heads. The 0-80 retard to 250 psi pressure range is used on dry systems where the lines are filled with air pressure until system activation. Wetted parts include a brass socket with “C” shaped bronze Bourdon tube. Units can withstand temperatures of –40 to 150˚F.

Complete details are available on our website @ http://www.dwyer-inst.com/htdocs/pressure/Series35WPrice.cfm
Dwyer Instruments, Inc.

P.O. Box 373
Michigan City, Ind., 46361
Phone: (800) 872-9141 or (219) 879-8000

Fax: (219) 872-9057

Internet address: www.dwyer-inst.com
E-mail: lit@dwyer-inst.com
_959145343.unknown

